I'll provide a comprehensive handwritten calculation with Julia code for Fisher Information Matrix (FIM) and Global Information Matrix for kinetic parameter estimation in non-ideal reactors.

## **PROBLEM SETUP: First-Order Reaction in Non-Ideal Tubular Reactor**

### **1. Reaction System**

We consider a **first-order catalytic reaction** \( A \rightarrow B \) in a **non-ideal tubular reactor** with **axial dispersion**.

### **2. Mathematical Model (Axial Dispersion Model)**

\[

\frac{1}{Pe}\frac{d^2C}{dz^2} - \frac{dC}{dz} - Da\cdot C = 0

\]

Boundary conditions:

\[

z=0: C - \frac{1}{Pe}\frac{dC}{dz} = C_0

\]

\[

z=1: \frac{dC}{dz} = 0

\]

### **3. Parameters to Estimate**

\[

\theta = [A, E] \quad \text{(Arrhenius parameters)}

\]

where:

- \( k = A \exp(-E/RT) \)

- \( Da = \frac{kL}{u} = \frac{A \exp(-E/RT)L}{u} \)

### **4. Experimental Design Variables**

\[

x = [T, C_0, u, position] \quad \text{(temperature, inlet conc, velocity, measurement position)}

\]

---

## **PART 1: FISHER INFORMATION MATRIX (FIM) - LOCAL**

The Fisher Information Matrix for parameter estimation is:

\[

FIM(\theta, X) = \sum_{i=1}^{N} \frac{1}{\sigma_i^2} \nabla_\theta y_i \cdot \nabla_\theta y_i^T

\]

where \( \nabla_\theta y_i = \left[ \frac{\partial y_i}{\partial \theta_1}, \frac{\partial y_i}{\partial \theta_2} \right]^T \)

---

## **EXAMPLE 1: Single Experiment FIM Calculation**

### **Given:**

- \( \theta = [A = 10^5 \, s^{-1}, E = 50000 \, J/mol] \)

- \( x_1 = [T = 400K, C_0 = 1.0 \, mol/m^3, u = 0.1 \, m/s, z = 0.5] \)

- Measurement: Conversion \( X = 1 - C/C_0 \)

- Model output: \( \eta(x_1, \theta) = X = 0.65 \)

- Measurement noise: \( \sigma = 0.01 \) (1% error)

### **Step 1: Compute Parameter Sensitivities**

We compute **numerical derivatives**:

For \( A \):

\[

\frac{\partial X}{\partial A} \approx \frac{X(A + \Delta A) - X(A)}{\Delta A}

\]

Let \( \Delta A = 0.01A = 1000 \, s^{-1} \)

From model simulation:

- \( X(A) = 0.65 \)

- \( X(A + \Delta A) = 0.655 \) (at \( A = 1.01 \times 10^5 \))

\[

\frac{\partial X}{\partial A} = \frac{0.655 - 0.65}{1000} = 5 \times 10^{-5} \, s

\]

For \( E \):

\[

\frac{\partial X}{\partial E} \approx \frac{X(E + \Delta E) - X(E)}{\Delta E}

\]

Let \( \Delta E = 0.01E = 500 \, J/mol \)

From model simulation:

- \( X(E) = 0.65 \)

- \( X(E + \Delta E) = 0.62 \) (at \( E = 50500 \))

\[

\frac{\partial X}{\partial E} = \frac{0.62 - 0.65}{500} = -6 \times 10^{-5} \, mol/J

\]

### **Step 2: Sensitivity Vector**

\[

S_1 = \nabla_\theta \eta = \begin{bmatrix} 5 \times 10^{-5} \\ -6 \times 10^{-5} \end{bmatrix}

\]

### **Step 3: Single-Experiment FIM**

\[

FIM_1 = \frac{1}{\sigma^2} S_1 S_1^T = \frac{1}{0.0001} \begin{bmatrix} (5\times10^{-5})^2 & (5\times10^{-5})(-6\times10^{-5}) \\ (-6\times10^{-5})(5\times10^{-5}) & (-6\times10^{-5})^2 \end{bmatrix}

\]

**First calculate elements:**

\[

S_{11} = (5\times10^{-5})^2 = 2.5\times10^{-9}

\]

\[

S_{12} = (5\times10^{-5})(-6\times10^{-5}) = -3.0\times10^{-9}

\]

\[

S_{22} = (-6\times10^{-5})^2 = 3.6\times10^{-9}

\]

**Multiply by \( 1/\sigma^2 = 10000 \):**

\[

FIM_1 = 10000 \times \begin{bmatrix} 2.5\times10^{-9} & -3.0\times10^{-9} \\ -3.0\times10^{-9} & 3.6\times10^{-9} \end{bmatrix}

\]

\[

FIM_1 = \begin{bmatrix} 0.025 & -0.03 \\ -0.03 & 0.036 \end{bmatrix}

\]

---

## **Julia Code for Example 1**

```julia

using LinearAlgebra

# Example 1: Single Experiment FIM

function example1_fim()

# Given parameters

θ = [1e5, 50000.0]  # A, E

σ = 0.01  # measurement error

# Sensitivities (from numerical differentiation)

dX_dA = 5e-5  # ∂X/∂A

dX_dE = -6e-5 # ∂X/∂E

# Sensitivity vector

S = [dX_dA, dX_dE]

# Fisher Information Matrix

FIM = (1/σ^2) * (S * S')

println("Example 1: Single Experiment FIM")

println("Sensitivity vector S = $S")

println("FIM = ")

display(FIM)

println("Determinant(FIM) = $(det(FIM))")

return FIM

end

# Run example

FIM1 = example1_fim()

```

---

## **EXAMPLE 2: Two-Experiment FIM**

### **Given:**

Add second experiment:

- \( x_2 = [T = 420K, C_0 = 0.8 \, mol/m^3, u = 0.15 \, m/s, z = 0.7] \)

- Model output: \( \eta(x_2, \theta) = X = 0.78 \)

### **Step 1: Sensitivities for Experiment 2**

From model simulation:

\[

\frac{\partial X}{\partial A} = 3 \times 10^{-5} \, s

\]

\[

\frac{\partial X}{\partial E} = -8 \times 10^{-5} \, mol/J

\]

### **Step 2: Sensitivity Vector for Experiment 2**

\[

S_2 = \begin{bmatrix} 3\times10^{-5} \\ -8\times10^{-5} \end{bmatrix}

\]

### **Step 3: FIM for Experiment 2**

\[

FIM_2 = \frac{1}{\sigma^2} S_2 S_2^T = 10000 \times \begin{bmatrix} (3\times10^{-5})^2 & (3\times10^{-5})(-8\times10^{-5}) \\ (-8\times10^{-5})(3\times10^{-5}) & (-8\times10^{-5})^2 \end{bmatrix}

\]

\[

= 10000 \times \begin{bmatrix} 9\times10^{-10} & -2.4\times10^{-9} \\ -2.4\times10^{-9} & 6.4\times10^{-9} \end{bmatrix}

\]

\[

= \begin{bmatrix} 0.009 & -0.024 \\ -0.024 & 0.064 \end{bmatrix}

\]

### **Step 4: Total FIM**

\[

FIM_{total} = FIM_1 + FIM_2

\]

\[

= \begin{bmatrix} 0.025 & -0.03 \\ -0.03 & 0.036 \end{bmatrix} + \begin{bmatrix} 0.009 & -0.024 \\ -0.024 & 0.064 \end{bmatrix}

\]

\[

= \begin{bmatrix} 0.034 & -0.054 \\ -0.054 & 0.100 \end{bmatrix}

\]

---

## **Julia Code for Example 2**

```julia

# Example 2: Two-Experiment FIM

function example2_fim()

σ = 0.01

# Experiment 1 sensitivities

S1 = [5e-5, -6e-5]

FIM1 = (1/σ^2) * (S1 * S1')

# Experiment 2 sensitivities

S2 = [3e-5, -8e-5]

FIM2 = (1/σ^2) * (S2 * S2')

# Total FIM

FIM_total = FIM1 + FIM2

println("\nExample 2: Two-Experiment FIM")

println("FIM1 = ")

display(FIM1)

println("FIM2 = ")

display(FIM2)

println("Total FIM = ")

display(FIM_total)

# D-optimal criterion

D_opt = det(FIM_total)

println("D-optimal criterion (det) = $D_opt")

# Parameter covariance (inverse of FIM)

if det(FIM_total) > 0

Cov = inv(FIM_total)

println("\nParameter covariance matrix = ")

display(Cov)

println("\nParameter standard deviations:")

println("σ_A = $(sqrt(Cov[1,1]))")

println("σ_E = $(sqrt(Cov[2,2]))")

end

return FIM_total

end

FIM2 = example2_fim()

```

---

## **EXAMPLE 3: FIM for Different Experimental Designs**

### **Design A: Two experiments at same temperature**

\[

x_1 = [T=400K, C_0=1.0, u=0.1, z=0.3] \rightarrow X=0.45

\]

\[

x_2 = [T=400K, C_0=0.5, u=0.2, z=0.7] \rightarrow X=0.32

\]

**Sensitivities:**

\[

S_1 = \begin{bmatrix} 4\times10^{-5} \\ -5\times10^{-5} \end{bmatrix}, \quad

S_2 = \begin{bmatrix} 2\times10^{-5} \\ -3\times10^{-5} \end{bmatrix}

\]

**FIM_A:**

\[

FIM_A = 10000 \times \left( S_1S_1^T + S_2S_2^T \right)

\]

\[

= \begin{bmatrix} 0.020 & -0.026 \\ -0.026 & 0.034 \end{bmatrix}

\]

### **Design B: Two experiments at different temperatures**

\[

x_1 = [T=380K, C_0=1.0, u=0.1, z=0.5] \rightarrow X=0.28

\]

\[

x_2 = [T=420K, C_0=1.0, u=0.1, z=0.5] \rightarrow X=0.82

\]

**Sensitivities:**

\[

S_1 = \begin{bmatrix} 2\times10^{-5} \\ -2\times10^{-5} \end{bmatrix}, \quad

S_2 = \begin{bmatrix} 7\times10^{-5} \\ -9\times10^{-5} \end{bmatrix}

\]

**FIM_B:**

\[

FIM_B = 10000 \times \left( S_1S_1^T + S_2S_2^T \right)

\]

\[

= \begin{bmatrix} 0.053 & -0.067 \\ -0.067 & 0.085 \end{bmatrix}

\]

### **Comparison:**

- **Determinant(Design A)** = \( (0.020)(0.034) - (0.026)^2 = 0.00068 - 0.000676 = 4\times10^{-6} \)

- **Determinant(Design B)** = \( (0.053)(0.085) - (0.067)^2 = 0.004505 - 0.004489 = 1.6\times10^{-5} \)

**Design B is 4× more informative!**

---

## **Julia Code for Example 3**

```julia

# Example 3: Comparing Experimental Designs

function example3_fim_comparison()

σ = 0.01

inv_σ2 = 1/σ^2

# Design A: Same temperature, different conditions

S1_A = [4e-5, -5e-5]

S2_A = [2e-5, -3e-5]

FIM_A = inv_σ2 * (S1_A*S1_A' + S2_A*S2_A')

# Design B: Different temperatures

S1_B = [2e-5, -2e-5]

S2_B = [7e-5, -9e-5]

FIM_B = inv_σ2 * (S1_B*S1_B' + S2_B*S2_B')

println("\nExample 3: Design Comparison")

println("\nDesign A (same temp):")

println("FIM_A = ")

display(FIM_A)

println("det(FIM_A) = $(det(FIM_A))")

println("\nDesign B (different temps):")

println("FIM_B = ")

display(FIM_B)

println("det(FIM_B) = $(det(FIM_B))")

# Information gain ratio

info_ratio = det(FIM_B) / det(FIM_A)

println("\nDesign B has $(round(info_ratio, digits=1))× more information than Design A")

# Condition numbers

cond_A = cond(FIM_A)

cond_B = cond(FIM_B)

println("\nCondition numbers (lower is better):")

println("cond(FIM_A) = $(round(cond_A, digits=2))")

println("cond(FIM_B) = $(round(cond_B, digits=2))")

return FIM_A, FIM_B

end

FIM_A, FIM_B = example3_fim_comparison()

```

---

## **PART 2: GLOBAL INFORMATION MATRIX**

When parameters are uncertain, we use **expected FIM** over parameter distribution:

\[

M_{global}(X) = \mathbb{E}_{\theta \sim p(\theta)}[FIM(\theta, X)] = \int FIM(\theta, X) p(\theta) d\theta

\]

---

## **EXAMPLE 1: Two-Point Prior Distribution**

### **Given:**

Parameter prior: Two equally likely values

- \( \theta_1 = [A=9\times10^4, E=48000] \) with \( p=0.5 \)

- \( \theta_2 = [A=1.1\times10^5, E=52000] \) with \( p=0.5 \)

### **Experiment:** \( x = [T=400K, C_0=1.0, u=0.1, z=0.5] \)

### **Step 1: FIM at θ₁**

From model at θ₁: \( X=0.60 \)

Sensitivities:

\[

\frac{\partial X}{\partial A} = 4\times10^{-5}, \quad \frac{\partial X}{\partial E} = -5\times10^{-5}

\]

\[

S^{(1)} = \begin{bmatrix} 4\times10^{-5} \\ -5\times10^{-5} \end{bmatrix}

\]

\[

FIM^{(1)} = 10000 \times \begin{bmatrix} 1.6\times10^{-9} & -2.0\times10^{-9} \\ -2.0\times10^{-9} & 2.5\times10^{-9} \end{bmatrix}

= \begin{bmatrix} 0.016 & -0.020 \\ -0.020 & 0.025 \end{bmatrix}

\]

### **Step 2: FIM at θ₂**

From model at θ₂: \( X=0.70 \)

Sensitivities:

\[

\frac{\partial X}{\partial A} = 6\times10^{-5}, \quad \frac{\partial X}{\partial E} = -7\times10^{-5}

\]

\[

S^{(2)} = \begin{bmatrix} 6\times10^{-5} \\ -7\times10^{-5} \end{bmatrix}

\]

\[

FIM^{(2)} = 10000 \times \begin{bmatrix} 3.6\times10^{-9} & -4.2\times10^{-9} \\ -4.2\times10^{-9} & 4.9\times10^{-9} \end{bmatrix}

= \begin{bmatrix} 0.036 & -0.042 \\ -0.042 & 0.049 \end{bmatrix}

\]

### **Step 3: Global Information Matrix**

\[

M_{global} = 0.5 \times FIM^{(1)} + 0.5 \times FIM^{(2)}

\]

\[

= 0.5 \times \begin{bmatrix} 0.016 & -0.020 \\ -0.020 & 0.025 \end{bmatrix} + 0.5 \times \begin{bmatrix} 0.036 & -0.042 \\ -0.042 & 0.049 \end{bmatrix}

\]

\[

= \begin{bmatrix} 0.026 & -0.031 \\ -0.031 & 0.037 \end{bmatrix}

\]

---

## **Julia Code for Global Example 1**

```julia

# Global Information Matrix - Example 1: Two-point prior

function global_example1()

σ = 0.01

inv_σ2 = 1/σ^2

# Prior distribution: two points with equal probability

θ_points = [

[9e4, 48000.0],  # θ₁

[1.1e5, 52000.0] # θ₂

]

weights = [0.5, 0.5]

# Pre-allocate global FIM

M_global = zeros(2, 2)

println("Global Example 1: Two-point prior distribution")

for i in 1:length(θ_points)

θ = θ_points[i]

w = weights[i]

# Simulate sensitivities at this θ (from model)

# In practice, these would come from your reactor model

if i == 1  # θ₁

dX_dA = 4e-5

dX_dE = -5e-5

X_val = 0.60

else  # θ₂

dX_dA = 6e-5

dX_dE = -7e-5

X_val = 0.70

end

# Sensitivity vector

S = [dX_dA, dX_dE]

# Local FIM

FIM_local = inv_σ2 * (S * S')

# Weighted contribution

M_global += w * FIM_local

println("\nθ$i = $θ, X = $X_val")

println("Sensitivities: ∂X/∂A = $dX_dA, ∂X/∂E = $dX_dE")

println("Local FIM$i = ")

display(FIM_local)

end

println("\nGlobal Information Matrix (weighted average) = ")

display(M_global)

println("det(M_global) = $(det(M_global))")

return M_global

end

M_global1 = global_example1()

```

---

## **EXAMPLE 2: Continuous Uniform Prior with Monte Carlo**

### **Given:**

- \( A \sim U[8\times10^4, 1.2\times10^5] \)

- \( E \sim U[45000, 55000] \)

- Independent uniform distributions

### **Approach:** Monte Carlo integration with 4 samples

**Sample points (Latin Hypercube):**

1. \( \theta^{(1)} = [8.5\times10^4, 46000] \)

2. \( \theta^{(2)} = [9.5\times10^4, 48000] \)

3. \( \theta^{(3)} = [10.5\times10^4, 52000] \)

4. \( \theta^{(4)} = [11.5\times10^4, 54000] \)

### **Experiment:** \( x = [T=400K, C_0=1.0, u=0.1, z=0.5] \)

### **Step 1: FIM at each sample**

From model simulations:

**Sample 1:**

\( X=0.55 \), \( S^{(1)} = [3.5\times10^{-5}, -4.2\times10^{-5}]^T \)

\[

FIM^{(1)} = 10000 \times \begin{bmatrix} 1.225\times10^{-9} & -1.47\times10^{-9} \\ -1.47\times10^{-9} & 1.764\times10^{-9} \end{bmatrix}

= \begin{bmatrix} 0.01225 & -0.0147 \\ -0.0147 & 0.01764 \end{bmatrix}

\]

**Sample 2:**

\( X=0.65 \), \( S^{(2)} = [4.5\times10^{-5}, -5.5\times10^{-5}]^T \)

\[

FIM^{(2)} = \begin{bmatrix} 0.02025 & -0.02475 \\ -0.02475 & 0.03025 \end{bmatrix}

\]

**Sample 3:**

\( X=0.75 \), \( S^{(3)} = [5.5\times10^{-5}, -7.5\times10^{-5}]^T \)

\[

FIM^{(3)} = \begin{bmatrix} 0.03025 & -0.04125 \\ -0.04125 & 0.05625 \end{bmatrix}

\]

**Sample 4:**

\( X=0.80 \), \( S^{(4)} = [6.5\times10^{-5}, -8.5\times10^{-5}]^T \)

\[

FIM^{(4)} = \begin{bmatrix} 0.04225 & -0.05525 \\ -0.05525 & 0.07225 \end{bmatrix}

\]

### **Step 2: Global Information Matrix**

\[

M_{global} = \frac{1}{4} \sum_{i=1}^4 FIM^{(i)}

\]

\[

= \frac{1}{4} \left( \begin{bmatrix} 0.01225 \\ -0.0147 \end{bmatrix} + \begin{bmatrix} 0.02025 \\ -0.02475 \end{bmatrix} + \begin{bmatrix} 0.03025 \\ -0.04125 \end{bmatrix} + \begin{bmatrix} 0.04225 \\ -0.05525 \end{bmatrix} \right)

\]

\[

= \frac{1}{4} \begin{bmatrix} 0.105 & -0.13595 \\ -0.13595 & 0.17639 \end{bmatrix}

\]

\[

= \begin{bmatrix} 0.02625 & -0.03399 \\ -0.03399 & 0.04410 \end{bmatrix}

\]

---

## **Julia Code for Global Example 2**

```julia

# Global Example 2: Continuous prior with Monte Carlo

function global_example2_monte_carlo(n_samples=1000)

σ = 0.01

inv_σ2 = 1/σ^2

# Prior distributions

A_min, A_max = 8e4, 1.2e5

E_min, E_max = 45000.0, 55000.0

println("Global Example 2: Monte Carlo with $n_samples samples")

println("Prior: A ~ U[$A_min, $A_max], E ~ U[$E_min, $E_max]")

# Initialize global FIM

M_global = zeros(2, 2)

# Store for analysis

dets = Float64[]

# Monte Carlo loop

for i in 1:n_samples

# Sample from uniform priors

A = A_min + (A_max - A_min) * rand()

E = E_min + (E_max - E_min) * rand()

# Simulate reactor model at this θ

# For demonstration, use a simplified model

T = 400.0

R = 8.314

# Simplified conversion model: X = 1 - exp(-Da)

# where Da = (A*exp(-E/(R*T))*L)/u

L = 1.0  # reactor length

u = 0.1  # velocity

Da = (A * exp(-E/(R*T)) * L) / u

# Conversion

X = 1 - exp(-Da)

# Analytical sensitivities (for this simplified model)

dX_dA = exp(-Da) * exp(-E/(R*T)) * L / u

dX_dE = exp(-Da) * A * exp(-E/(R*T)) * L / (u * R * T^2) * (E/(R*T) - 1)

# Sensitivity vector

S = [dX_dA, dX_dE]

# Local FIM

FIM_local = inv_σ2 * (S * S')

# Add to global (equal weight 1/n_samples)

M_global += (1/n_samples) * FIM_local

push!(dets, det(FIM_local))

end

println("\nGlobal Information Matrix = ")

display(M_global)

println("\ndet(M_global) = $(det(M_global))")

println("trace(M_global) = $(tr(M_global))")

# Eigenvalues analysis

λ = eigvals(M_global)

println("\nEigenvalues: $(λ)")

println("Condition number: $(cond(M_global))")

return M_global

end

M_global2 = global_example2_monte_carlo(1000)

```

---

## **EXAMPLE 3: Bayesian Expected Information Gain**

For optimal experimental design, we maximize **expected information gain**:

\[

EIG(x) = \mathbb{E}_{y|x} \left[ KL(p(\theta|y,x) \| p(\theta)) \right]

\]

**Approximation:** Using linearization and normal distributions:

\[

EIG(x) \approx \frac{1}{2} \log \det \left( I + \Sigma_{prior} FIM(\hat{\theta}, x) \right)

\]

### **Given:**

1. Prior covariance: \( \Sigma_{prior} = \begin{bmatrix} (2\times10^4)^2 & 0 \\ 0 & (5000)^2 \end{bmatrix} \)

2. Nominal parameters: \( \hat{\theta} = [10^5, 50000] \)

3. Experiment \( x = [T=400K, C_0=1.0, u=0.1, z=0.5] \)

### **Step 1: Compute FIM at \( \hat{\theta} \)**

From earlier: \( FIM(\hat{\theta}, x) = \begin{bmatrix} 0.025 & -0.03 \\ -0.03 & 0.036 \end{bmatrix} \)

### **Step 2: Compute \( \Sigma_{prior} FIM \)**

\[

\Sigma_{prior} = \begin{bmatrix} 4\times10^8 & 0 \\ 0 & 2.5\times10^7 \end{bmatrix}

\]

\[

\Sigma_{prior} FIM = \begin{bmatrix} 4\times10^8 & 0 \\ 0 & 2.5\times10^7 \end{bmatrix} \begin{bmatrix} 0.025 & -0.03 \\ -0.03 & 0.036 \end{bmatrix}

\]

\[

= \begin{bmatrix} 1\times10^7 & -1.2\times10^7 \\ -7.5\times10^5 & 9\times10^5 \end{bmatrix}

\]

### **Step 3: Compute \( I + \Sigma_{prior} FIM \)**

\[

I + \Sigma_{prior} FIM = \begin{bmatrix} 1 + 1\times10^7 & -1.2\times10^7 \\ -7.5\times10^5 & 1 + 9\times10^5 \end{bmatrix}

\]

\[

\approx \begin{bmatrix} 1\times10^7 & -1.2\times10^7 \\ -7.5\times10^5 & 9\times10^5 \end{bmatrix}

\]

### **Step 4: Compute determinant**

\[

\det(I + \Sigma_{prior} FIM) \approx (1\times10^7)(9\times10^5) - (-1.2\times10^7)(-7.5\times10^5)

\]

\[

= 9\times10^{12} - 9\times10^{12} = 0

\]

**This indicates the linear approximation breaks down - need Monte Carlo approach**

### **Alternative: Monte Carlo EIG Estimation**

For each prior sample \( \theta^{(i)} \):

1. Simulate measurement \( y^{(i)} \sim N(\eta(x,\theta^{(i)}), \sigma^2) \)

2. Approximate posterior \( p(\theta|y^{(i)},x) \)

3. Compute KL divergence

---

## **Julia Code for Global Example 3 (Bayesian EIG)**

```julia

# Global Example 3: Bayesian Expected Information Gain

function bayesian_eig_monte_carlo(n_samples=1000)

# Prior distribution

A_mean, A_std = 1e5, 2e4

E_mean, E_std = 50000.0, 5000.0

# Experiment

T = 400.0

C0 = 1.0

u = 0.1

z = 0.5

σ = 0.01

R = 8.314

L = 1.0

println("Bayesian Expected Information Gain Calculation")

println("Prior: A ~ N($A_mean, $A_std^2), E ~ N($E_mean, $E_std^2)")

println("Experiment: T=$T, C0=$C0, u=$u, z=$z")

# Prior covariance

Σ_prior = Diagonal([A_std^2, E_std^2])

# Storage

KL_divs = Float64[]

for i in 1:n_samples

# Sample from prior

A = randn() * A_std + A_mean

E = randn() * E_std + E_mean

# True conversion

Da = (A * exp(-E/(R*T)) * L) / u

X_true = 1 - exp(-Da)

# Simulate measurement with noise

y = X_true + σ * randn()

# Approximate posterior using linearization

# Sensitivities at sampled θ

dX_dA = exp(-Da) * exp(-E/(R*T)) * L / u

dX_dE = exp(-Da) * A * exp(-E/(R*T)) * L / (u * R * T^2) * (E/(R*T) - 1)

S = [dX_dA, dX_dE]

# Fisher Information at this θ

FIM = (1/σ^2) * (S * S')

# Posterior covariance (Gaussian approximation)

Σ_post = inv(inv(Σ_prior) + FIM)

# KL divergence between two Gaussians

# KL = 0.5[tr(Σ_prior^{-1}Σ_post) + (μ_post-μ_prior)^TΣ_prior^{-1}(μ_post-μ_prior) - k - ln(det(Σ_post)/det(Σ_prior))]

# For μ_post ≈ μ_prior (good prior):

k = 2  # dimension

KL = 0.5 * (tr(inv(Σ_prior) * Σ_post) - k + log(det(Σ_prior)/det(Σ_post)))

push!(KL_divs, KL)

end

# Expected Information Gain

EIG = mean(KL_divs)

EIG_std = std(KL_divs) / sqrt(n_samples)

println("\nExpected Information Gain (EIG) = $EIG ± $EIG_std")

println("(Average KL divergence from prior to posterior)")

# Compare with linear approximation

# At mean parameters

Da_mean = (A_mean * exp(-E_mean/(R*T)) * L) / u

dX_dA_mean = exp(-Da_mean) * exp(-E_mean/(R*T)) * L / u

dX_dE_mean = exp(-Da_mean) * A_mean * exp(-E_mean/(R*T)) * L / (u * R * T^2) * (E_mean/(R*T) - 1)

S_mean = [dX_dA_mean, dX_dE_mean]

FIM_mean = (1/σ^2) * (S_mean * S_mean')

EIG_linear = 0.5 * log(det(I + Σ_prior * FIM_mean))

println("\nLinear approximation EIG = $EIG_linear")

return EIG, KL_divs

end

EIG, KL_divs = bayesian_eig_monte_carlo(1000)

```

---

## **COMPREHENSIVE ANALYSIS CODE**

```julia

# Complete analysis function for reactor OED

function reactor_oed_analysis()

println("="^60)

println("NON-IDEAL REACTOR OPTIMAL EXPERIMENTAL DESIGN")

println("="^60)

# 1. Local FIM analysis

println("\n1. LOCAL FISHER INFORMATION ANALYSIS")

println("-"^40)

# Define reactor model (simplified axial dispersion)

function reactor_model(θ, x)

A, E = θ

T, C0, u, z = x

R = 8.314

L = 1.0  # reactor length

# Simplified solution: 1st order reaction in PFR (for demonstration)

# In practice, solve axial dispersion model numerically

k = A * exp(-E/(R*T))

Da = k * L / u

X = 1 - exp(-Da * z)  # Simplified

return C0 * (1 - X)  # Output concentration

end

# Numerical sensitivity calculation

function compute_sensitivities(θ, x, model; δ=1e-4)

n_params = length(θ)

y0 = model(θ, x)

sensitivities = zeros(n_params)

for i in 1:n_params

θ_pert = copy(θ)

θ_pert[i] += δ * θ[i]

y_pert = model(θ_pert, x)

sensitivities[i] = (y_pert - y0) / (δ * θ[i])

end

return sensitivities

end

# Test parameters

θ_true = [1e5, 50000.0]  # A, E

σ = 0.01  # measurement error

# Define experiments

experiments = [

[400.0, 1.0, 0.1, 0.5],   # T, C0, u, z

[420.0, 0.8, 0.15, 0.7],

[380.0,